# Final Project: Machine Learning for AEInnova Devices

Context document for the Final Year Project (TFG) on monitoring data from Motors and Steam traps (Purgadores) in projects using AEInnova devices. It includes the exploration of the DatasetI and DatasetZ datasets.

## 0. Summary of available data (exploration)

Two line-delimited JSON files (DynamoDB-style format) have been analysed: DatasetI and DatasetZ.

| Dataset | Total records | Unique devices |
| --- | --- | --- |
| DatasetI | 87,057 | 5 |
| DatasetZ | 105,096 | 7 |
| Total | 192,153 | 10 (some appear in both projects) |

Per-device detail (data points, variables) is given in sections 3.1 and 3.2.

## 1. Device types and variables

### 1.1 Motors

| Device | Variables measured |
| --- | --- |
| InduEye VibroSense 3 | Vibration and Temperature (T1) |

- The motor unit used in the projects is NOD-0007 (InduEye VibroSense 3).

- Operating mode (mode): It is critical to take it into account. The device has several operating modes and the frequency range measured depends on the mode: depending on the mode, the unit sweeps one vibration spectrum range or another.

- Physical axis (axis): The axis being measured (sensor orientation, e.g. x, y, z) also matters. Vibration on one axis is not the same as on another.

- Readings (spectrum data): Newer devices send readings: many points of the vibration spectrum over a frequency range that varies with operating mode. In the past, aggregated magnitudes px/py/pz and fpx/fpy/fpz were sent; new devices send readings (spectrum) instead. For the TFG it is advisable to use only devices that send readings, as they represent the current product evolution.

### 1.2 Steam traps (Purgadores)

Two product families, with different sensors:

| Family | Name | Variables measured |
| --- | --- | --- |
| LS | LeackSense (LS) | Ultrasound and Temperatures (T1, T2) |
| TS | TempSense (TS) | Temperatures only (T1 and T2) |

## 2. Equipment variants and evolution

At AEInnova, equipment is continuously improved. There are:

- New versions that send more useful information or different operating modes.

- Multiple variants of each device type (by board model, firmware, etc.).

This means the datasets may contain:

- Different board types (e.g. NOD-0007, DUTNOD-0003, DUTNOD-0004, POCIETSTLTT, POCIELSTL3TTU).

- Different firmware versions.

- Older (OLD) units vs newer versions (Super OK, 2.0), with possible differences in data volume and quality.

For the TFG it is important to document which device/variant corresponds to each time series or equipment ID.

## 3. Projects and datasets

Data from two projects is available: DatasetI and DatasetZ.

### 3.1 DatasetI

Total in DatasetI: 87,057 records, 5 devices.

#### Motor (NOD-0007, InduEye VibroSense 3)

| Device ID | Notes | Model | Product | Firmware | Points | Variables |
| --- | --- | --- | --- | --- | --- | --- |
| 70B3D5326007D0DA | Good candidate (many points, readings) | NOD-0007 | InduEye VibroSense 3 | Orchid_01.26.hex | 37,897 | 31: T1, T2, x, y, z, px1–pz3, fpx1–fpz3, mode, meta_fcnt, gateways_signal, timezone, Po, axis, readings |
| 70B3D5326007D0FE | Good candidate (many points, readings) | NOD-0007 | InduEye VibroSense 3 | Orchid_01.26.hex | 28,432 | 12: T1, T2, x, y, z, mode, meta_fcnt, gateways_signal, timezone, axis, readings |
| 70B3D5326007D0D9 | Few data (readings but few points) | NOD-0007 | InduEye VibroSense 3 | — | 269 | 10: T1, T2, x, y, z, axis, mode, meta_fcnt, readings |

- In the past, motors sent px/py/pz (power per axis) and fpx/fpy/fpz (peak frequencies); new devices send readings (full spectrum). In the dataset, 70B3D5326007D0DA still includes px/py/pz and fpx/fpy/fpz in addition to readings; 70B3D5326007D0FE and 70B3D5326007D0D9 send x, y, z and readings. For the TFG it is preferable to rely only on data from devices with readings (all listed ones have them) and treat the readings format as the reference for the current product.

- For the TFG (motors): Pay close attention to operating mode and physical axis: the frequency range of the readings changes with mode, and the axis determines which direction is being measured. Do not mix data from different modes or axes in the same model without normalising or labelling correctly.

- The exploration shows T1/T2 ranges with anomalous values (e.g. T1 outside −50…200 °C); they should be filtered or flagged in preprocessing.

#### Steam trap (DUTNOD-0003, LS: US + 2×T)

| Device ID | Notes | Model | Product | Firmware | Points | Variables |
| --- | --- | --- | --- | --- | --- | --- |
| 1750323479398620 | OLD (only 8 months). Good candidate (many points) | DUTNOD-0003 | LS | — | 11,927 | 9: T1, T2, peak_freq, peak_value, rms, saturation, mode, meta_fcnt |
| 0C5032345F395920 | Good candidate (many points) | DUTNOD-0003 | LS 2.0 | — | 8,532 | 13: T1, T2, Po, Pulses, pulses, RMS(20–30 kHz), RMS(30–50 kHz), diag, mode, meta_fcnt, gateways_signal, timezone |

### 3.2 DatasetZ

Total in DatasetZ: 105,096 records, 7 devices. Only steam traps (TS and LS); no motors in this project.

#### Steam trap TS (2×T only: T1 and T2)

| Device ID | Notes | Model | Product | Firmware | Points | Variables |
| --- | --- | --- | --- | --- | --- | --- |
| 8C1F6458B0006465 | Super OK. Good candidate (many points) | DUTNOD-0004 | TS | — | 34,439 | 11: T1, T2, meta_fcnt, mode, peak_freq, peak_value, rms, saturation, gateways_signal, timezone |
| 70B3D5326007D066 | OLD but OK. Good candidate (many points) | POCIETSTLTT | TS | — | 25,088 | 8: T1, T2, d3, d4, d5, d6, meta_fcnt |
| 70B3D5326007D061 | OLD but OK. Few points | POCIETSTLTT | TS | — | 133 | 8: T1, T2, d3–d6, meta_fcnt |
| 8C1F6458B000641C | OLD but OK. Few points | NOD-0004 | TS | — | 681 | 4: T1, T2, meta_fcnt, timestamp |

- Some TS devices (e.g. 8C1F6458B0006465) also send ultrasound magnitudes (peak_freq, rms, saturation) due to firmware shared with LS by mistake. They are not suitable for the projects.

- POCIETSTLTT units use d3–d6 channels instead of ultrasound; 8C1F6458B000641C only T1, T2 and metadata. They are not suitable for the projects.

#### Steam trap LS (US + 2×T)

| Device ID | Notes | Model | Product | Firmware | Points | Variables |
| --- | --- | --- | --- | --- | --- | --- |
| 8C1F6458B0006422 | Super OK. Good candidate (many points) | DUTNOD-0003 | LS 2.0 | — | 29,598 | 19: T1, T2, Po, Pulses, pulses, RMS(20–30 kHz), RMS(30–50 kHz), peak_freq, peak_value, rms, rms_high, rms_low, saturation, diag, mode, meta_fcnt, gateways_signal, timezone |
| 70B3D5326007D068 | OLD and NO. Good candidate (many points) | POCIELSTL3TTU | LS | — | 14,993 | 11: T1, T2, d3–d6, meta_fcnt, x, y, z |
| 1750323479398620 | OLD, other machine. Few points | DUTNOD-0003 | LS | — | 164 | 8: T1, T2, peak_freq, peak_value, rms, saturation, meta_fcnt |

- The same 1750323479398620 appears in DatasetI (11,927 points) and in DatasetZ (164 points from another machine); do not mix series without a project/machine indicator.

## 4. Exploration notes (for preprocessing and ML)

- Schema heterogeneity: Not all devices of the same type send the same variables (e.g. one motor with 31 variables vs another with 12).

- Motors: mode and axis: Operating mode determines the frequency range in which readings (spectrum) are measured; the axis (axis) indicates which physical direction is being measured.

- Anomalous T1/T2 values: The exploration shows very wide ranges or impossible values (e.g. −500 °C, +500 °C). In preprocessing: use reasonable thresholds, filtering or “sensor not connected” labelling.

- Devices with few points: 70B3D5326007D0D9 (269), 70B3D5326007D061 (133), 8C1F6458B000641C (681), 1750323479398620 in DatasetZ (164). Decide whether to use them for training, validation or analysis only.

- Timestamps in milliseconds: Convert to date/time to analyse seasonality, shifts or sensor drift.

## 5. Summary for the TFG

- Motors (InduEye VibroSense 3, NOD-0007)

- Variables: vibration (x, y, z; in the past also px/py/pz, fpx/fpy/fpz) and readings (full spectrum). New devices send readings; the old format was px/py/pz and fpx/fpy/fpz. For the TFG it is advisable to use only those that send readings, as they represent the current evolution.

- Essential: (a) Operating mode (mode): the frequency range measured varies with mode; (b) Axis measured (axis): the physical axis (x, y, z) matters. Readings are spectrum data over a range that depends on mode and axis; they must be handled accordingly (segment by mode/axis or include mode/axis as features).

- In DatasetI: 3 motor devices (37,897 + 28,432 + 269 points); all send readings.

- Steam traps (Purgadores)

- LS: ultrasound (rms, peak, saturation; in LS 2.0 also RMS bands, Pulses, Po) + T1 + T2.

- TS: T1 + T2 (some with peak/rms/saturation due to firmware error).

- Identify in each record whether it is LS or TS to choose features correctly (e.g. do not use ultrasound in pure TS series).

- Equipment heterogeneity

- Different models (NOD-0007, DUTNOD-0003, DUTNOD-0004, POCIETSTLTT, POCIELSTL3TTU).

- OLD vs newer versions (LS 2.0, “Super OK”).

- This can affect normalisation, anomaly detection and model generalisation.

- Datasets

- DatasetI: 87,057 records; 3 NOD-0007 motors, 2 LS steam traps (DUTNOD-0003).

- DatasetZ: 105,096 records; steam traps only (TS and LS), 7 devices; include device and type (TS/LS) metadata in the ML pipeline.

With this information, the scope of the TFG (which devices, variables and projects to use) can be defined and preprocessing and modelling decisions documented.